# Embedded Systems Lab Programs

Exp No: 3

# Stepper Motor using AT89C51 using Proteus

## Program:

ORG 0000H

UP: MOV P2,#09H

ACALL DELAY

MOV P2,#0CH

ACALL DELAY

MOV P2,#06H

ACALL DELAY

MOV P2,#03H

ACALL DELAY

SJMP UP

DELAY: MOV R4,#18

H1:    MOV R3,#255

H2:    DJNZ R3,H2

DJNZ R4,H1

RET

END

## Output:

Exp No: 4

# 7 Segment Display using AT89C51 using Proteus

## Program:

ORG 000H

UP: MOV P2,#0C0H

ACALL DELAY

MOV P2,#0F9H

ACALL DELAY

MOV P2,#0A4H

ACALL DELAY

MOV P2,#0B0H

ACALL DELAY

MOV P2,#99H

ACALL DELAY

MOV P2,#92H

ACALL DELAY

MOV P2,#82H

ACALL DELAY

MOV P2,#0F8H

ACALL DELAY

MOV P2,#80H

ACALL DELAY

MOV P2,#90H

ACALL DELAY

DELAY: MOV R5,#10

H1:    MOV R4,#180

H2:    MOV R3,#255

H3:    DJNZ R3,H3

DJNZ R4,H2

DJNZ R5,H1

RET

END

## Output:

Exp No: 9

# Arduino Based Manual Electronic Counter using Proteus

## Program:

int x0,x1,x2,x3,x4,x5,x6,x7,x8,x9;

int delay_time = 200;

void setup() {

// configure pin2 as an input and enable the internal pull-up resistor

pinMode(12, INPUT_PULLUP);

pinMode(1, OUTPUT);

pinMode(2, OUTPUT);

pinMode(3, OUTPUT);

pinMode(4, OUTPUT);

pinMode(5, OUTPUT);

pinMode(6, OUTPUT);

pinMode(7, OUTPUT);

}

void loop() {

// read the pushbutton value into a variable

int sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x0 = true;

}

while (x0) {

zero();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x1 = true;

x0 = false;

}

}

while (x1) {

one();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x2 = true;

x1 = false;

}

}

}

Note: the source file's code ends here — the remaining while-loop blocks (for x2 through x9) and closing braces were not present in the original document and may need to be completed.

## Output: