LAB REPORT

COURSE CODE: ECA1423

COURSE NAME: EMBEDDED SYSTEMS

NAME: D Annie Shirlyn

Reg.No: 192511319

DEPARTMENT: B.E – CSE

GIT HUB LINK: https://github.com/Annie Shirlyn D/192511319.Emedded System

SIMATS ENGINEERING

Saveetha Institute of Medical and Technical Sciences

September – 2026

| Exp. No | Date | Name of the Experiment | Page. No |
| --- | --- | --- | --- |
| 1 | 22/07/26 | Flashing of Led using AT89C51 Microcontroller using Proteus |  |
| 2 | 22/07/26 | Generation of Square Wave using Proteus |  |
| 3 | 24/07/26 | Stepper Motor Using AT89C51 using Proteus |  |
| 4 | 24/07/26 | 7 Segment Display Using AT89C51 using Proteus |  |
| 5 | 27/07/26 | Arduino based Manual Electronic Counter using Proteus |  |
| 6 | 27/07/26 | Arduino to 16x2 LCD Display using Proteus |  |
| 7 | 29/07/26 | Serial Communication using Arduino with Proteus |  |
| 8 | 29/07/26 | Gas Sensor MQ-2 in Proteus using Arduino |  |
| 9 | 31/07/26 | Ultrasonic Sensor in Proteus using Arduino |  |
| 10 | 03/08/26 | PIR Sensor in Proteus using Arduino |  |
| 11 | 05/08/26 | Study of GPIO & Bit Addressing in AT89C51 using Embedded C |  |
| 12 | 07/08/26 | Embedded C program to multiply two 16-bit binary numbers. |  |
| 13 | 10/08/26 | Embedded C program to add an array of 16-bit numbers and store the 32-bit result in internal RAM. |  |
| 14 | 10/08/26 | Embedded C program to display “Hello World” message |  |
| 15 | 12/08/26 | Embedded C program to convert the hexadecimal data to decimal and display the digits on ports P0, P1 and P2 |  |
| 16 | 14/08/26 | Serial Interrupt Using UART (Keyboard Input) In Keil |  |
| 17 | 14/08/26 | UART Interfacing using Interrupt in KEIL |  |
| 18 | 17/08/26 | Generation of PWM using TIMER Interrupt |  |
| 19 | 21/08/26 | Blinking LEDs using software delay routine in LPC2148 kit |  |

| 20 | 21/08/26 | Program to read the switch and display in the LEDs using LPC2148 kit |  |
| --- | --- | --- | --- |
| 21 | 24/08/26 | Display a number in seven segment LED in LPC2148 kit |  |
| 22 | 24/08/26 | Serial transmission and reception using on-chip UART in LPC2148 kit |  |
| 23 | 24/08/26 | Access an internal ADC and display the binary output in LEDs in LPC2148 kit |  |
| 24 | 26/08/26 | Modeling Clock-Driven Concurrent Processes Using SC_THREAD and wait() in SystemC |  |
| 25 | 26/08/26 | Producer–Consumer Problem Using SystemC |  |
| 26 | 28/08/26 | Transaction Level Modeling (TLM) using SystemC |  |
| 27 | 28/08/26 | Binary Semaphore in SystemC |  |
| 28 | 28/08/26 | Counting Semaphore in SystemC |  |
| 29 | 28/08/26 | Priority-Based Interrupt Handling |  |
| 30 | 28/08/26 | RTOS Task Scheduling |  |

EXPERIMENT 1:

PROGRAM:

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT:

EXPERIMENT 2:

PROGRAM:

ORG 0000H

UP: SETB P2.0

ACALL DELAY

CLR P2.0

ACALL DELAY

SJMP UP

DELAY: MOV R4,#35

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT:

EXPERIMENT 3:

PROGRAM:

ORG 0000H

UP: MOV P2,#09H

ACALL DELAY

MOV P2,#0CH

ACALL DELAY

MOV P2,#06H

ACALL DELAY

MOV P2,#03H

ACALL DELAY

SJMP UP

DELAY:MOV R4,#18

H1:MOV R3,#255

H2:DJNZ R3,H2

DJNZ R4,H1

RET

END

OUTPUT:

EXPERIMENT 4:

PROGRAM:

ORG 000H

UP:MOV P2,#0C0H

ACALL DELAY

MOV P2,#0F9H

ACALL DELAY

MOV P2,#0A4H

ACALL DELAY

MOV P2,#0B0H

ACALL DELAY

MOV P2,#99H

ACALL DELAY

MOV P2,#92H

ACALL DELAY

MOV P2,#82H

ACALL DELAY

MOV P2,#0F8H

ACALL DELAY

MOV P2, #80H

ACALL DELAY

MOV P2,#90H

ACALL DELAY

DELAY: MOV R5,#10

H1:MOV R4,#180

H2:MOV R3,#255

H3:DJNZ R3,H3

DJNZ R4,H2

DJNZ R5,H1

RET

END

OUTPUT:

EXPERIMENT 5:

PROGRAM:

int x0,x1,x2,x3,x4,x5,x6,x7,x8,x9;

int delay_time=200;

void setup() {

//configure pin2 as an input and enable the internal pull-up resistor

pinMode(12, INPUT_PULLUP);

pinMode(1,OUTPUT);

pinMode(2,OUTPUT);

pinMode(3,OUTPUT);

pinMode(4,OUTPUT);

pinMode(5,OUTPUT);

pinMode(6,OUTPUT);

pinMode(7,OUTPUT);

}

void loop() {

//read the pushbutton value into a variable

int sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x0=true;

}

while(x0){

zero();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x1=true;

x0=false;

}

}

while(x1){

one();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x2=true;

x1=false;

}

}

while(x2){

two();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x3=true;

x2=false;

}

}

while(x3){

three();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x4=true;

x3=false;

}

}

while(x4){

four();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x5=true;

x4=false;

}

}

while(x5){

five();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x6=true;

x5=false;

}

}

while(x6){

six();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x7=true;

x6=false;

}

}

while(x7){

seven();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x8=true;

x7=false;

}

}

while(x8){

eight();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x9=true;

x8=false;

}

}

while(x9){

nine();

sensorVal = digitalRead(12);

if (sensorVal == LOW) {

x0=true;

x9=false;

}

}

}

void zero()

{

for(int i=1;i<7;i++)

{

digitalWrite(i,HIGH);

digitalWrite(7,LOW);

}

delay(delay_time);

}

void one()

{

digitalWrite(1,LOW);

digitalWrite(2,HIGH);

digitalWrite(3,HIGH);

digitalWrite(4,LOW);

digitalWrite(5,LOW);

digitalWrite(6,LOW);

digitalWrite(7,LOW);

delay(delay_time);

}

void two()

{

digitalWrite(1,HIGH);

digitalWrite(2,HIGH);

digitalWrite(3,LOW);

digitalWrite(4,HIGH);

digitalWrite(5,HIGH);

digitalWrite(6,LOW);

digitalWrite(7,HIGH);

delay(delay_time);

}

void three()

{

digitalWrite(1,HIGH);

digitalWrite(2,HIGH);

digitalWrite(3,HIGH);

digitalWrite(4,HIGH);

digitalWrite(5,LOW);

digitalWrite(6,LOW);

digitalWrite(7,HIGH);

delay(delay_time);

}

void four()

{

digitalWrite(1,LOW);

digitalWrite(2,HIGH);

digitalWrite(3,HIGH);

digitalWrite(4,LOW);

digitalWrite(5,LOW);

digitalWrite(6,HIGH);

digitalWrite(7,HIGH);

delay(delay_time);

}

void five()

{

digitalWrite(1,HIGH);

digitalWrite(2,LOW);

digitalWrite(3,HIGH);

digitalWrite(4,HIGH);

digitalWrite(5,LOW);

digitalWrite(6,HIGH);

digitalWrite(7,HIGH);

delay(delay_time);

}

void six()

{

digitalWrite(1,HIGH);

digitalWrite(2,LOW);

digitalWrite(3,HIGH);

digitalWrite(4,HIGH);

digitalWrite(5,HIGH);

digitalWrite(6,HIGH);

digitalWrite(7,HIGH);

delay(delay_time);

}

void seven()

{

digitalWrite(1,HIGH);

digitalWrite(2,HIGH);

digitalWrite(3,HIGH);

digitalWrite(4,LOW);

digitalWrite(5,LOW);

digitalWrite(6,LOW);

digitalWrite(7,LOW);

delay(delay_time);

}

void eight()

{

digitalWrite(1,HIGH);

digitalWrite(2,HIGH);

digitalWrite(3,HIGH);

digitalWrite(4,HIGH);

digitalWrite(5,HIGH);

digitalWrite(6,HIGH);

digitalWrite(7,HIGH);

delay(delay_time);

}

void nine()

{

digitalWrite(1,HIGH);

digitalWrite(2,HIGH);

digitalWrite(3,HIGH);

digitalWrite(4,HIGH);

digitalWrite(5,LOW);

digitalWrite(6,HIGH);

digitalWrite(7,HIGH);

delay(delay_time);

}

OUTPUT:

EXPERIMENT 6:

PROGRAM:

Arduino to 16x2 LCD Display using Proteus

#include <LiquidCrystal.h>

LiquidCrystal lcd (7, 8, 9, 10, 11, 12);

// your PWM numbers may be different

void setup() {

lcd.begin(16, 2);

lcd.print("put your message here");

}

void loop () {

for (int i=0; 1<13; i++)

}

lcd.scrollDisplayLeft();

delay(700);

}

}

OUTPUT:

EXPERIMENT 7:

PROGRAM:

void setup()

{

Serial.begin(9600); // send and receive at 9600 baud

}

int number = 0;

void loop()

{

Serial.print("Number is ");

Serial.println(number); // print the number

delay(500); // delay half second between numbers

number++; // to the next number

}

OUTPUT:

EXPERIMENT 8:

PROGRAM:

#define GreenLed 13

#define Sensor A0

void setup(){

pinMode(13,OUTPUT);

pinMode(12,OUTPUT);

pinMode(A0,INPUT);

Serial.begin(9600);

}

void loop() {

int value = analogRead(A0);

Serial.print("Analogic Value coming from the sensor : ");

Serial.println(value);

delay(100);

if(value>600){

digitalWrite(13,HIGH);

}

else{

digitalWrite(13,LOW);

}

delay(20);

}

Code for MQ6 sensor:

int mq6Pin = A0; // MQ-5 analog pin connected to A0

void setup() {

Serial.begin(9600);

}

void loop() {

int sensorValue = analogRead(mq5Pin);

Serial.print("MQ5 Sensor Value: ");

Serial.println(sensorValue);

delay(1000); // 1-second delay

}

OUTPUT:

EXPERIMENT 9:

PROGRAM:

// Program Sketch

int trig = 10;

int echo = 9;

long duration;

int cm;

void setup() {

// put your setup code here, to run once:

pinMode(trig, OUTPUT);

pinMode(echo,INPUT);

Serial.begin(9600);

}

void loop() { // Sketch to calculate ultrasonic distance

digitalWrite(trig, LOW);

delayMicroseconds(10);

digitalWrite(trig, HIGH);

delayMicroseconds(10);

digitalWrite(trig, LOW);

delayMicroseconds(10);

duration = pulseIn(echo, HIGH);

cm = (duration/2) * 0.034;

// Print Value on Serial Monitor

Serial.print(" Distance = ");

Serial.print(cm);

Serial.println(" cm");

}

OUTPUT:

EXPERIMENT 10:

PROGRAM:

int Status = 13; // Digital pin D6

int sensor = 12; // Digital pin D7

void setup() {

Serial.begin(9600);

pinMode(sensor, INPUT); // declare sensor as input

pinMode(Status, OUTPUT); // declare LED as output

}

void loop() {

long state = digitalRead(sensor);

Serial.println(state);

if(state == HIGH) {

digitalWrite (Status, HIGH);

Serial.println("Motion detected!");

delay(1000);

}

else {

digitalWrite (Status, LOW);

Serial.println("Motion absent!");

delay(1000);

}

}

OUTPUT:

EXPERIMENT 11:

PROGRAM:

#include <reg51.h>

sbit LED1 = P1^0;

sbit LED2 = P1^1;

void delay()

{

unsigned int i,j;

for(i=0;i<200;i++)

for(j=0;j<1000;j++);

}

void main()

{

while(1)

{

LED1 = 1;

LED2 = 0;

delay();

LED1 = 0;

LED2 = 1;

delay();

}

}

OUTPUT:

EXPERIMENT 12:

PROGRAM:

#include <reg51.h> void main()

{

while (1)

{

unsigned int num1, num2; unsigned long int product; num1 = 0x2222;

num2 = 0xBBBB;

product = (unsigned long int)num1 * num2; P0= product &0xff;

P1= (product & 0xff00)>>8;

P2 =(product & 0xff0000)>>16; P3 =(product & 0xff000000)>>24;

}

}

OUTPUT:

EXPERIMENT 13:

PROGRAM:

#include <reg51.h>

void main(void)

{

unsigned int i;

array[00x00 unsigned int array[5] = {0x1111, 0x2222, 0x8888, 0x4444, 0xABCD};

unsigned long sum = 0;

for (i = 0; i < 5; i++)

{

sum = sum + array[i];

}

P0 = (unsigned char)(sum & 0xFF); // Lower 8 bits

P1 = (unsigned char)((sum >> 8) & 0xFF); // Next 8 bits

P2 = (unsigned char)((sum >> 16) & 0xFF); // Upper 8 bits

while(1); // keep output stable

}

OUTPUT:

EXPERIMENT 14:

PROGRAM:

#include <reg51.h> #include <stdio.h>

void main (void){

SCON = 0x50; /* SCON: mode 1, 8-bit UART, enable rcvr */ TMOD = 0x20;

/* TMOD: timer 1, mode 2, 8-bit reload */ TH1 = 0xFD; /* TH1: reload

value for 9600 baudrate */

TR1 = 1; /* TR1: timer 1 run */

TI = 1; /* TI: set TI to send first char of UART */ while (1)

{

printf ("Hello World ! \n ");

}

}

OUTPUT:

EXPERIMENT 15:

PROGRAM:

#include <reg51.h>

void main(void)

{

unsigned char hexa = 0x08;

unsigned char hundreds, tens, units;

ACC = hexa;

B = 10;

ACC = ACC / B;

units = B;

B = 10;

ACC = ACC / B;

tens = B;

hundreds = ACC;

P0 = units;

P1 = tens;

P2 = hundreds;

while(1);

}

OUTPUT:

EXPERIMENT :16

PROGRAM:

#include <reg51.h>

#include <stdio.h>

void serial_ISR(void) interrupt 4

{

char ch;

if (RI)

{

RI = 0; // Clear receive interrupt flag

ch = SBUF; // Read received character

printf("\n>>> INTERRUPT RECEIVED: ");

printf("%c\n", ch);

}

}

void delay(void)

{

unsigned int i;

for(i = 0; i < 50000; i++); // Delay for visibility

}

void main(void)

{

SCON = 0x50; // UART mode 1, 8-bit, REN enabled

TMOD = 0x20; // Timer1 mode 2 (auto-reload)

TH1 = 0xFD; // Baud rate 9600

TR1 = 1; // Start Timer1

IE = 0x90; // EA = 1, ES = 1 (enable serial interrupt)

TI = 1; // Enable printf

while(1)

{

printf("Hello World\n");

delay();

}

}

OUTPUT:

EXPERIMENT 17:

PROGRAM:

#include <reg51.h>

void serial_ISR(void) interrupt 4

{

if(RI)

{

P1 = SBUF;

RI = 0;

}

}

void main()

{

TMOD=0x20;

TH1=0xFD;

SCON=0x50;

TR1=1;

ES=1;

EA=1;

while(1);

}

OUTPUT:

EXPERIMENT 18:

PROGRAM:

#include <reg51.h>

sbit PWM = P1^0;

unsigned char count = 0;

unsigned char duty = 100; // Change this value (0–200)

void timer0_ISR(void) interrupt 1

{

TH0 = 0xFC; // Reload for 1ms delay

TL0 = 0x66;

count++;

if(count < duty)

PWM = 1; // ON time

else

PWM = 0; // OFF time

if(count >= 200) // Total period = 200ms

count = 0;

}

void main()

{

TMOD = 0x01; // Timer0 Mode 1 (16-bit)

TH0 = 0xFC; // 1ms preload

TL0 = 0x66;

IE = 0x82; // EA = 1, ET0 = 1

TR0 = 1; // Start Timer0

while(1);

}

OUTPUT:

EXPERIMENT:19

PROGRAM:

#include "lpc214x.h"

void delay (unsigned int k);

void main(void)

{

IODIR0 = 0xFFFFFFFF; //Configure Port0 as output Port

PINSEL0 = 0; //Configure Port0 as General Purpose IO

while(1)

{

IOSET0 = 0x0000ff00; //Set P0.15-P0.8 to '1'

delay(1000); //1 sec Delay

IOCLR0 = 0x0000ff00; //Set P0.15-P0.8 to '0'

delay(1000); //1 Sec Delay

}

}

//Delay Program

//Input - delay value in milli seconds

void delay(unsigned int k)

{

unsigned int i,j;

for (j=0; j<k; j++)

for(i = 0; i<=800; i++);

}

OUTPUT:

EXPERIMENT 20:

PROGRAM:

#include "lpc214x.h"

int main(void)

{

unsigned int sw_sts;

IODIR0 = 0x0000ff00; //Configure Port0

PINSEL0 = 0; //Configure Port0 as General Purose IO

while(1)

{

sw_sts = IOPIN0;

IOSET0 = 0x0000ff00; //Set P0.15-P0.8 to '1'

IOCLR0 = sw_sts >> 8; //Set P0.15-P0.8 to '0'

}

}

OUTPUT:

EXPERIMENT 21:

PROGRAM:

#include <LPC214X.H>

#define DS3 1<<13 // P0.13

#define DS4 1<<12 // P0.12

#define SEG_CODE 0xFF<<16 // Segment Data from P0.16 to P0.23

unsigned char const seg_dat[]={0x3F, 0x6, 0x5B, 0x4F, 0x66, 0x6D, 0x7D, 0x7, 0x7F,

0x67};

void delayms(int n)

{

int i,j;

for(i=0;i<n;i++)

{for(j=0;j<5035;j++) //5035 for 60Mhz ** 1007 for 12Mhz

{;}

}

}

int main (void)

{

unsigned char count;

PINSEL0 = 0; // Configure Port0 as General Purpose IO => P0.0 to P0.15

PINSEL1 = 0; // Configure Port0 as General Purpose IO => P0.16 to P0.31

IODIR0 = SEG_CODE | DS3 | DS4; //Configure Segment data & Select signal as output

IOSET0 = SEG_CODE | DS3 ; //Disable DS3 display

IOCLR0 = DS4; //Enable DS4 Display

count = 0; //Initialize Count

//Display Count value

IOCLR0 = SEG_CODE;

IOSET0 = seg_dat[count]<<16;

while(1)

{

delayms(1000); //1 sec delay

count++; //Increment count

if(count>9) count=0; //Limit 0-9

//Display Count value

IOCLR0 = SEG_CODE;

IOSET0 = seg_dat[count]<<16;

}

}

OUTPUT:

EXPERIMENT 22:

PROGRAM:

#include <lpc214x.h>

void UART0_Init(void)

{

PLL0CON = 0;

PLL0FEED=0xAA;

PLL0FEED=0x55;

VPBDIV = 1;

// Fpclk = 12.000.000 MHz

// DLM,DLH = Fpclk / (19200*16) = 39 = 0x27

PINSEL0 |= 0x5; // Select UART0 RXD/TXD

U0FCR = 0; // Disable FIFO's

U0LCR = 0x83; // 8N1, enable Divisor latch bit

U0DLL = 0x27; // baud rate fixed to 19200 @ PCLK = 12 Mhz

U0DLM = 0;

U0LCR = 3; // Disable Divisor latch bit

}

/*------------------------------------------------------------------*/

/* Function to send one char. to Serial Port */

void sout(unsigned char dat1)

{

while(!(U0LSR & 0x20));//Wait for Tx Buffer Empty

U0THR = dat1; //Send to UART1

}

/*-------------------------------------------------------*/

int main (void)

{ int dat;

UART0_Init();

do

{

if(U0LSR & 1) /* Check for RDR (Receiver Data Ready)command */

{

dat = U0RBR;// Receive Data from Srial Port

sout(dat); // Send Data to Srial Port

}

}while(1);

}

OUTPUT:

EXPERIMENT 23:

PROGRAM:

#include <LPC214X.H>

#define LEDS 0xFF<<8 //LED => P0.8 to P0.15

/////////////////////////////////////////

/*--- ADC Signal Declaration */

/////////////////////////////////////////

#define AD0_1 1<< 24

#define CLK_DIV 1<<8

#define PDN 1<<21

#define SOC 1<<24

#define BURST 1<<16

#define DONE 1<<31

/*-----------------------------------------------------------*/

//Delay Program

//Input - delay value in milli seconds

void delay(unsigned int k)

{

unsigned int i,j;

for (j=0; j<k; j++)

for(i = 0; i<=800; i++);

}

/*-----------------------------------------------------------*/

void adc_init()

{

unsigned long int ADC_CH;

ADC_CH = 0 | 1 << 1; //Channel AD0.1

AD0CR = SOC | PDN | CLK_DIV | ADC_CH | BURST ;

}

/*-----------------------------------------------------------*/

unsigned int adc_read( unsigned char channel)

{

unsigned int aval;

unsigned long int val;

if (channel == 1) val = AD0DR1;

else if (channel == 2) val = AD0DR2;

else if (channel == 3) val = AD0DR3;

val = val >> 6;

val = val & 0x3FF;

aval = val;

return (aval);

}

/*-----------------------------------------------------------*/

//////////////////////////

/*----Main Program------*/

//////////////////////////

int main(void)

{

unsigned int tp1;

IODIR0 = LEDS; //Configure Port0 as output Port

PINSEL0 = 0; //Configure Port0 as General Purpose IO

PINSEL1 = 0 | AD0_1; // Enable AD0.1

adc_init(); //Initialise on-chip ADC

do

{ tp1 = adc_read(1); // Channel AD0 0.1

tp1 = tp1 >> 2; // ADC 10 bit But LED 8bit, Truncate lsb 2 bits

IOSET0 = LEDS; //Switch OFF all LEDS

IOCLR0 = tp1 << 8; //Set VAlue

delay(1000);

}while(1);

}

OUTPUT:

EXPERIMENT 24:

PROGRAM:

#include <systemc.h>

SC_MODULE(ClockExample) {

sc_in<bool> clk;

void process() {

while (true) {

wait();

std::cout << "Time: " << sc_time_stamp()

<< " - Clock Triggered" << std::endl;

}

}

SC_CTOR(ClockExample) {

SC_THREAD(process);

sensitive << clk.pos();

}

};

int sc_main(int argc, char* argv[]) {

sc_clock clock("clock", 1, SC_NS);

ClockExample module("module");

module.clk(clock);

sc_start(5, SC_NS);

return 0;

}

OUTPUT:

EXPERIMENT25:

PROGRAM:

#include <systemc.h>

// Producer Module

SC_MODULE(Producer) {

sc_fifo_out<int> out;

void produce() {

for (int i = 1; i <= 5; i++) {

cout << "Produced: " << i << endl;

out.write(i);

wait(1, SC_SEC);

}

}

SC_CTOR(Producer) {

SC_THREAD(produce);

}

};

// Consumer Module

SC_MODULE(Consumer) {

sc_fifo_in<int> in;

void consume() {

int value;

while (true) {

in.read(value);

cout << "Consumed: " << value << endl;

wait(1, SC_SEC);

}

}

SC_CTOR(Consumer) {

SC_THREAD(consume);

}

};

int sc_main(int argc, char* argv[]) {

sc_fifo<int> fifo(5);

Producer producer("Producer");

Consumer consumer("Consumer");

producer.out(fifo);

consumer.in(fifo);

sc_start(10, SC_SEC);

return 0;

}

OUTPUT:

EXPERIMENT 26:

PROGRAM:

#include <systemc>

#include <tlm>

#include <tlm_utils/simple_initiator_socket.h>

#include <tlm_utils/simple_target_socket.h>

using namespace sc_core;

using namespace tlm;

using namespace std;

// Target

struct Target : sc_module {

tlm_utils::simple_target_socket<Target> socket;

SC_CTOR(Target) : socket("socket") {

socket.register_b_transport(this, &Target::b_transport);

}

void b_transport(tlm_generic_payload& trans, sc_time& delay) {

int* data = (int*) trans.get_data_ptr();

cout << "Target received: " << *data << endl;

delay += sc_time(10, SC_NS);

}

};

// Initiator

struct Initiator : sc_module {

tlm_utils::simple_initiator_socket<Initiator> socket;

SC_CTOR(Initiator) : socket("socket") {

SC_THREAD(thread_process);

}

void thread_process() {

tlm_generic_payload trans;

sc_time delay = SC_ZERO_TIME;

int data = 555;

trans.set_data_ptr(reinterpret_cast<unsigned char*>(&data));

trans.set_data_length(sizeof(data));

cout << "Initiator sending: " << data << endl;

socket->b_transport(trans, delay);

wait(delay);

}

};

int sc_main(int argc, char* argv[]) {

Initiator initiator("initiator");

Target target("target");

initiator.socket.bind(target.socket);

sc_start();

return 0;

}

OUTPUT:

EXPERIMENT 27:

PROGRAM:

#include <systemc.h>

SC_MODULE(binary_semaphore) {

int sem;

sc_event ev;

void task1() {

while(true) {

wait(1, SC_SEC);

if(sem == 1) {

sem = 0;

cout << "Task1 Entered Critical Section at " << sc_time_stamp() << endl;

wait(2, SC_SEC);

sem = 1;

ev.notify();

}

else {

wait(ev);

}

}

}

void task2() {

while(true) {

wait(2, SC_SEC);

if(sem == 1) {

sem = 0;

cout << "Task2 Entered Critical Section at " << sc_time_stamp() << endl;

wait(1, SC_SEC);

sem = 1;

ev.notify();

}

else {

wait(ev);

}

}

}

SC_CTOR(binary_semaphore) {

sem = 1;

SC_THREAD(task1);

SC_THREAD(task2);

}

};

int sc_main(int argc, char* argv[]) {

binary_semaphore obj("Binary_Semaphore");

sc_start(10, SC_SEC);

return 0;

}

OUTPUT:

EXPERIMENT 28:

PROGRAM:

#include <systemc.h>

SC_MODULE(counting_semaphore) {

int sem;

void task() {

while(true) {

wait(1, SC_SEC);

if(sem > 0) {

sem--;

cout << "Task Accessing Resource at "

<< sc_time_stamp()

<< " Remaining: " << sem << endl;

wait(2, SC_SEC);

sem++;

}

}

}

SC_CTOR(counting_semaphore) {

sem = 2;

SC_THREAD(task);

SC_THREAD(task);

SC_THREAD(task);

}

};

int sc_main(int argc, char* argv[]) {

counting_semaphore obj("Counting");

sc_start(10, SC_SEC);

return 0;

}

OUTPUT:

EXPERIMENT 29:

PROGRAM:

#include <systemc.h>

SC_MODULE(priority_interrupt) {

sc_event low_int, high_int;

void low_ISR() {

while(true) {

wait(low_int);

cout << "Low Priority Interrupt at "

<< sc_time_stamp() << endl;

wait(3, SC_SEC);

}

}

void high_ISR() {

while(true) {

wait(high_int);

cout << "High Priority Interrupt at "

<< sc_time_stamp() << endl;

wait(1, SC_SEC);

}

}

void generator() {

wait(1, SC_SEC);

low_int.notify();

wait(1, SC_SEC);

high_int.notify();

}

SC_CTOR(priority_interrupt) {

SC_THREAD(low_ISR);

SC_THREAD(high_ISR);

SC_THREAD(generator);

}

};

int sc_main(int argc, char* argv[]) {

priority_interrupt obj("Priority_INT");

sc_start(10, SC_SEC);

return 0;

}

OUTPUT:

EXPERIMENT 30:

PROGRAM:

#include <systemc.h>

SC_MODULE(rtos_scheduler) {

void task1() {

while(true) {

cout << "Task1 Running at "

<< sc_time_stamp() << endl;

wait(1, SC_SEC);

}

}

void task2() {

while(true) {

cout << "Task2 Running at "

<< sc_time_stamp() << endl;

wait(1, SC_SEC);

}

}

SC_CTOR(rtos_scheduler) {

SC_THREAD(task1);

SC_THREAD(task2);

}

};

int sc_main(int argc, char* argv[]) {

rtos_scheduler obj("Scheduler");

sc_start(6, SC_SEC);

return 0;

}

OUTPUT: